# CHAPTER 3

# ⚙️ PHARMACEUTICAL PROCESSES AND METHODS

### (Unit Operations + Classical Pharmaceutical Operations)

## 3.1 WHAT THE SYLLABUS MEANS HERE

📌 PHARMACEUTICAL PROCESS — any physical or chemical operation carried out on a drug or excipient in order to convert it into a finished, stable, elegant and accurately dosed medicinal product.

📌 UNIT OPERATION — a basic physical step that is common to many manufacturing processes (e.g. size reduction, mixing, drying). "Unit operation" = physical change; "unit process" = chemical change (nitration, esterification, oxidation).

⭐ THE MASTER LIST — memorise this skeleton first, then fill in details

| # | Unit operation | Purpose in one line |
| --- | --- | --- |
| 1 | Size reduction (comminution) | Make particles smaller |
| 2 | Size separation (sieving/ classification) | Sort particles by size |
| 3 | Mixing (blending) | Make a uniform mass |
| 4 | Drying (desiccation) | Remove moisture |
| 5 | Evaporation | Concentrate by removing solvent as vapour |
| 6 | Distillation | Separate & purify liquids by boiling + condensing |
| 7 | Sublimation | Solid → vapour → solid |
| 8 | Extraction | Remove desired constituent with a solvent (menstruum) |
| 9 | Filtration & clarification | Separate solids from liquids |
| 10 | Centrifugation | Separate by centrifugal force |
| 11 | Crystallisation | Obtain pure solid from solution |
| 12 | Sterilisation | Destroy all micro-organisms |
| 13 | Heat transfer | Move heat (conduction/ convection/ radiation) |
| 14 | Humidification/ dehumidification & air conditioning | Control moisture in air |
| 15 | Emulsification & suspension | Disperse one phase in another |
| 16 | Compression/ granulation/ coating | Tablet manufacture steps |

🧠 MEMORY HOOK — "Small Sorted Mixtures Dry, Evaporate, Distil, Sublime, Extract, Filter, Centrifuge, Crystallise, Sterilise."

## 3.2 SIZE REDUCTION (COMMINUTION)

📌 DEFINITION — Size reduction, also called comminution, grinding, milling, pulverisation or trituration, is the mechanical process of breaking a substance into smaller particles without altering its chemical nature.

### ⭐ Objectives / advantages of size reduction

- Increases surface area → faster dissolution & better bioavailability (e.g. griseofulvin, spironolactone).

- Increases rate of extraction of active constituents from crude drugs.

- Improves mixing/ blending uniformity of powders.

- Improves drying rate (larger surface).

- Permits accurate dosing of potent drugs (uniform small particles).

- Improves flow, texture & elegance (dusting powders, ointments, suspensions).

- Reduces sedimentation rate in suspensions (Stokes' law → smaller r, slower settling).

- Aids absorption & smoothness of topical products (no grittiness).

### ⚠️ Disadvantages / limitations

- Heat generation → degrades thermolabile drugs (waxes, gums, vitamins).

- Contamination from mill wear (metal).

- Loss of drug as dust; dust hazard/ explosion (e.g. starch, sulphur).

- Excessive fines → poor flow, caking, aggregation.

- Change in polymorphic form/ crystallinity.

- Segregation & static charge.

### Mechanisms (forces) of size reduction ⭐

| Force | Mechanism | Mill using it |
| --- | --- | --- |
| Impact (hammering) | Sharp instantaneous blow | Hammer mill, disintegrator |
| Compression (crushing) | Slow steady pressure | End-runner mill, Edge-runner mill, roller mill |
| Attrition (rubbing/ shearing) | Rubbing between surfaces | Colloid mill, roller mill, disc/ attrition mill |
| Cutting | Sharp blade | Cutter mill, guillotine |
| Impact + Attrition | Both together | Ball mill, fluid energy mill |

🧠 "I-C-A-C": Impact, Compression, Attrition, Cutting.

### Factors affecting size reduction

- Hardness (Mohs' scale) & abrasiveness.

- Toughness/ elasticity/ friability — tough materials resist; brittle break easily.

- Fibrous nature (needs cutting, not impact).

- Moisture content — ⭐ dry grinding needs < 5 % moisture; wet grinding > 50 %. Intermediate moisture causes caking/ balling.

- Softening/ melting point (heat-sensitive → cool, or use cryogenic grinding with liquid nitrogen/ dry ice).

- Stickiness, hygroscopicity, thermolability, purity required.

- Bulk density & feed rate; ratio of feed size to product size.

### 🔢 Laws of size reduction (energy laws)

| Law | Statement | Formula (E = energy) |
| --- | --- | --- |
| Rittinger's law | Energy required ∝ new surface area produced — best for fine grinding | E = K_R (1/d₂ − 1/d₁) |
| Kick's law | Energy required ∝ log of size reduction ratio — best for coarse grinding | E = K_K log(d₁/d₂) |
| Bond's law | Work is proportional to the square root of the surface : volume ratio — intermediate; introduces Work Index | E = 2K_B (1/√d₂ − 1/√d₁) |

🧠 "Rittinger = fine (Ritt for Really tiny), Kick = coarse, Bond = in Between."

### 📋 MILLS — the exam table

| Mill | Principle/ force | Feed & product size | Suitable for | Notes |
| --- | --- | --- | --- | --- |
| Mortar & Pestle | Attrition + compression (hand trituration) | — | Small-scale dispensing | Glass, porcelain, Wedgwood (rough — best for grinding), metal |
| Cutter mill | Cutting by rotating knives | Coarse | Fibrous, tough, dry & crude vegetable drugs, roots, barks | Screen controls size |
| Roller mill / Triple roller mill | Compression + attrition between rollers | — | Ointments, pastes, creams, cocoa, colours | Reduces grittiness |
| Hammer mill | Impact (high-speed swinging hammers) | 5–10 mm feed → 20–300 mesh | Almost any material — brittle, crystalline, fibrous, sticky (with cooling) | Very versatile; screen size sets product size; high heat |
| Ball mill | Impact + Attrition — balls tumble in a rotating cylinder | Coarse → very fine (as low as 10 µm) | Hard, abrasive, brittle drugs; can be used for wet & dry, sterile & closed grinding | ⭐ Critical speed = speed at which the balls are held to the wall by centrifugal force → no grinding. Optimum = 50–80 % (≈ 2/3) of critical speed. Balls 30–50 % of mill volume |
| Fluid energy mill (micronizer/ jet mill) | Impact + attrition by high-velocity air/ inert gas stream; particles collide with each other | → 1–20 µm (micronising) | Thermolabile & very fine powders; sterile & aseptic milling; antibiotics, corticosteroids | ⭐ No moving parts → no heat, no contamination; expansion of gas cools the chamber |
| Colloid mill | Shear/ attrition between a fixed stator & high-speed rotor | → colloidal size | Suspensions, emulsions, ointments (wet milling) | For liquids/ slurries only |
| End-runner mill | Compression + shear (steel pestle in revolving mortar) | Moderate | Crystalline & brittle drugs; small-scale | "Mechanical mortar & pestle" |
| Edge-runner mill | Compression + shear (heavy stone/ metal wheels revolving on a bed) | Moderately fine | Tough, fibrous & crystalline drugs; sticky masses | Two heavy rollers |
| Disintegrator/ Attrition (Disc) mill | Attrition between two grooved discs | Fine | Cereals, dry drugs |  |
| Pin/ Universal mill | Impact & attrition between pins | Fine | Versatile |  |
| Vibration mill | Impact & attrition, vibrating balls | Very fine | Hard materials | 60–80 % filled |
| Cryogenic grinding | Brittleness induced with liquid N₂ / dry ice | Fine | Thermolabile, waxy, gummy, volatile-oil-rich drugs | Preserves volatile oil |

⭐ HIGH-YIELD ONE-LINERS

- Best mill for a thermolabile drug / for micronising / for sterile products → Fluid energy mill.

- Best mill for fibrous crude vegetable drugs → Cutter mill.

- Mill that can grind under closed & sterile conditions and produce colloidal fineness → Ball mill.

- Mill for ointments & pastes → Triple roller mill / Colloid mill.

- Most versatile/ general purpose mill → Hammer mill.

- Critical speed formula: Nc = 1 / (2π) × √(g / (R − r)) (revolutions per second), where R = radius of mill, r = radius of ball.

### Classical (dispensing-bench) size-reduction terms ⭐⭐

| Term | 📌 Meaning |
| --- | --- |
| Trituration | Continuous rubbing/ grinding of a solid in a mortar with a pestle to reduce its size or to mix powders |
| Levigation | Reducing particle size by grinding a solid with a small amount of a liquid (levigating agent) in which it is insoluble, to make a smooth paste — e.g. zinc oxide + liquid paraffin/ glycerin for ointments |
| Pulverisation by intervention | Size reduction with the help of an additional volatile substance that is later removed — e.g. camphor + a few drops of alcohol/ ether ("spirit of wine"), iodine + ether; also making a fine powder of a gummy substance |
| Elutriation | Separating fine from coarse particles by suspending in a stream of water or air, fine ones being carried over |
| Comminution | The general term for size reduction |
| Granulation | Building small particles into larger free-flowing granules (wet or dry) |
| Spatulation | Mixing small quantities of powders on a tile/ slab with a spatula — used for eutectic/ liquefiable mixtures (no pressure, no trituration) |
| Sifting/ Bolting | Passing powders through a sieve or bolting cloth to mix & separate |
| Tumbling | Mixing powders by rotating them in a closed container |
| Geometric dilution | ⭐ Method of mixing a small quantity of a potent drug with a large quantity of diluent by doubling up at each stage |

## 3.3 SIZE SEPARATION (SIEVING, CLASSIFICATION)

📌 DEFINITION — Size separation (also classification, screening, sifting, grading) is the unit operation in which particles of a mixture are separated into two or more fractions according to their size.

### Objectives

- To obtain a powder of the official degree of fineness.

- To ensure uniform particle size → uniform dosage, good flow & mixing.

- To remove oversize/ fines & recycle them.

- To determine particle size distribution (sieve analysis).

### ⭐ Methods of size separation

| Method | Principle | Examples/ equipment |
| --- | --- | --- |
| Sieving (screening) | Particles pass through mesh apertures by gravity/ agitation | Standard sieves, vibratory sieve shaker, oscillating/ reciprocating sieve, rotary/ trommel sieve, agitation sieve, brushing sieve, centrifugal sieve |
| Sedimentation (in liquid) | Different settling velocities (Stokes' law) | Andreasen pipette, elutriation tank |
| Elutriation | Upward flow of fluid carries fines away | Air/ water elutriator |
| Air separation (cyclone/ classifier) | Centrifugal + gravitational force in an air stream | Cyclone separator, double-cone classifier, air separator |
| Magnetic separation | Removes iron contamination | Magnetic separators |
| Flotation | Difference in wettability/ density | Froth flotation |

🔢 Stokes' Law (used in sedimentation & suspension stability)

v = d² (ρs − ρl) g / 18 η

v = sedimentation velocity, d = particle diameter, ρs = density of solid, ρl = density of liquid, g = gravity, η = viscosity. ⭐ Sedimentation is slowed by ↓ particle size, ↑ viscosity, ↓ density difference.

### ⭐⭐ SIEVE / POWDER FINENESS — the most examined table

📌 Sieve number (mesh number) = number of apertures (meshes) per linear inch of the sieve. ⭐ Higher sieve number → smaller aperture → finer powder.

IP sieve numbers & nominal aperture sizes 🔢

| IP Sieve No. | Nominal mesh aperture |
| --- | --- |
| 10 | 1.7 mm |
| 22 | 710 µm |
| 44 | 355 µm |
| 60 | 250 µm |
| 85 | 180 µm |
| 120 | 125 µm |
| 150 | 105 µm |
| 180 | 90 µm |
| 240 | 63 µm |
| 300 | 53 µm |

⭐ IP GRADES OF POWDER (learn word-for-word)

| Grade of powder | Definition (IP) |
| --- | --- |
| Coarse powder (10/44) | All particles pass sieve no. 10 and not more than 40 % pass sieve no. 44 |
| Moderately coarse (22/60) | All pass no. 22, NMT 40 % pass no. 60 |
| Moderately fine (44/85) | All pass no. 44, NMT 40 % pass no. 85 |
| Fine powder (85) | All particles pass sieve no. 85 |
| Very fine powder (120) | All particles pass sieve no. 120 |
| Microfine (Superfine) | 90 % of particles NMT 50 µm |
| Superfine | 90 % of particles NMT 10 µm |

🧠 MEMORY HOOK — the "double" rule: the coarse grades are pairs where the second number is roughly double the first: 10/44, 22/60, 44/85 — then single numbers 85 (fine) and 120 (very fine).

⚠️ TRAP — For coarse/ moderately coarse/ moderately fine powders, two sieve numbers are quoted; for fine & very fine powders, only one.

BP/ classical descriptive grades

| Term | Passes through |
| --- | --- |
| Coarse | No. 10 |
| Moderately coarse | No. 22 |
| Moderately fine | No. 44 |
| Fine | No. 85 |
| Very fine | No. 120 |

### Standards of sieves & sieve analysis

- Sieves are made of wire cloth (brass, bronze, stainless steel) woven, not twilled; IP specifies aperture size, wire diameter, approximate screening area & tolerances.

- Sieve analysis — a nest/ stack of sieves arranged with the coarsest on top and a collecting pan at the bottom; shaken for a fixed time (e.g. 20 min); the retained weight on each sieve gives the particle size distribution.

- Results expressed as cumulative % oversize/ undersize curves; arithmetic & geometric mean diameter.

- Sieving of hygroscopic powders is done in a dry (dehumidified) room; air-jet sieving is used for very fine powders.

## 3.4 MIXING (BLENDING)

📌 DEFINITION — Mixing is the process in which two or more ingredients in a separate or roughly mixed condition are treated so that each particle of one lies as nearly as possible in contact with a particle of the other.

📌 Positive mixture — components mix spontaneously & irreversibly (miscible liquids, gases); no energy needed. 📌 Negative mixture — components tend to separate (suspensions, emulsions); needs continuous energy input. 📌 Neutral mixture — components neither mix spontaneously nor separate (pastes, ointments, mixed powders).

### Objectives of mixing

Uniform distribution of active drug (content uniformity), initiation of physical/ chemical reactions, dispersion of a phase (emulsification/ suspension), promotion of heat & mass transfer, and elegant appearance.

### Mechanisms of mixing (solids)

- Convective mixing — transfer of groups of particles to another location (ribbon blender).

- Shear mixing — slipping of layers past one another.

- Diffusive mixing — random motion of individual particles (tumbling mixers).

⚠️ Segregation (demixing) occurs when particles differ in size, shape or density; percolation of fines and trajectory segregation are common causes.

### 📋 Mixing equipment

| Class | Equipment | Best for |
| --- | --- | --- |
| Solid–solid (powder) mixers | Double-cone blender, Twin-shell (V-cone) blender, Cube/ drum tumbler, Ribbon blender, Sigma blade mixer, Planetary mixer, Nauta (conical screw) mixer, Fluidised bed mixer | Dry powders & granules |
| Liquid mixers (low viscosity) | Propeller (axial), Turbine (radial), Paddle agitators; air jet; baffled tanks to prevent vortex | Solutions, syrups, low-viscosity suspensions |
| Semisolid mixers | Sigma blade (Z-blade), Planetary, Triple roller mill, Colloid mill | Ointments, creams, pastes |
| Homogenisers/ emulsifiers | Silverson mixer emulsifier, colloid mill, ultrasonic homogeniser, high-pressure (valve) homogeniser | Emulsions & fine suspensions |
| Small-scale dispensing | Mortar & pestle (trituration), spatula on tile (spatulation), sieving, tumbling in a bag/ bottle | Extemporaneous powders |

⭐ Rules for mixing powders on the dispensing bench

- Use a mortar & pestle for coarse or hard powders; a tile & spatula for eutectics.

- Mix in ascending order of bulk density, i.e. begin with the powder of smallest bulk (never put the heavy powder in first — it stays at the bottom).

- Always use geometric dilution for potent (small-quantity) drugs.

- Coloured/ dye-like ingredients are sandwiched between portions of the diluent so that the colour acts as an indicator of complete mixing.

- Avoid excessive pressure on liquefiable/ eutectic mixtures and explosive mixtures.

- Sift through a sieve (no. 44/60) after mixing to break aggregates, then remix.

- Mix the smallest quantity first with an equal quantity of the other.

📌 GEOMETRIC DILUTION (Doubling-up) — worked example: Prepare 60 g of a powder containing 60 mg of a potent drug. Step 1: Triturate 60 mg drug + 60 mg diluent = 120 mg. Step 2: add 120 mg diluent = 240 mg → Step 3: + 240 mg = 480 mg → continue doubling (0.96 g → 1.92 g → 3.84 g → 7.68 g → 15.36 g → 30.72 g → 60 g). ⭐ The principle: at every stage add a quantity of diluent equal to the quantity already in the mortar.

### Evaluation of mixing

- Mixing index / index of mixing, coefficient of variation (CV) of sampled potency.

- Standard deviation of drug content of random samples; the lower the SD, the better.

- Use of a sampling thief at different points & depths.

- Scale of scrutiny — the smallest amount of the mix that will be examined (for tablets, one tablet weight).

## 3.5 DRYING (DESICCATION)

📌 DEFINITION — Drying is the removal of small amounts of a liquid (usually water) from a material by the application of heat, so that the liquid is removed as vapour.

⚠️ TRAP — Drying vs Evaporation:

- Drying removes a small amount of liquid → the product is a SOLID.

- Evaporation removes a large amount of liquid → the product is a LIQUID (concentrate).

### Objectives of drying

Prevent hydrolysis & microbial growth (stability), reduce bulk & transport cost, improve flow of granules, allow accurate weighing, make grinding possible, produce a definite crystalline/ powder form, meet pharmacopoeial moisture limits.

### ⭐ Moisture terminology

| Term | Meaning |
| --- | --- |
| Bound moisture | Water held in chemical/ physical combination — exerts vapour pressure lower than pure water; hard to remove |
| Unbound (free) moisture | Water in excess of equilibrium moisture content; exerts full vapour pressure |
| Equilibrium Moisture Content (EMC) | Moisture retained by the solid when in equilibrium with air of a given temp & humidity — cannot be removed at those conditions |
| Free moisture | Total moisture − EMC → the moisture that CAN be removed |
| Critical Moisture Content (CMC) | Moisture content at the point where the constant-rate period ends and the falling-rate period begins |
| Loss on drying (LOD) | % weight lost on drying, based on wet weight |
| Moisture content (dry basis) | Based on dry weight |

### ⭐ Drying rate curve — the three periods

Rate
 │        ┌──────────┐  ← B–C  CONSTANT RATE period
 │       /            \        (free surface water evaporates;
 │  A–B /              \        surface temp = wet-bulb temp)
 │ warm-up             \  C–D  FIRST FALLING RATE
 │                      \      (surface partially dry)
 │                       \ D–E SECOND FALLING RATE
 │                        \    (internal diffusion controls)
 └──────────────────────────── Moisture content →
                    ↑ CMC at point C

⭐ In the constant-rate period, drying is controlled by external conditions (air temp, humidity, velocity). In the falling-rate period, it is controlled by internal movement (diffusion) of moisture within the solid.

### 📋 DRYERS — the exam table

| Dryer | Principle | Suitable for | Key exam point |
| --- | --- | --- | --- |
| Tray (shelf/ compartment/ truck) dryer | Hot air circulated over material on trays (convection) | Granules, crude drugs, powders, small batches | Simple & cheap; long drying time (12–24 h), non-uniform; labour-intensive |
| Fluidised bed dryer (FBD) | Hot air blown upwards through the bed → particles float ("fluidised") | ⭐ Granules for tablets | Very rapid (20–40 min), uniform, thermal efficiency high; risk of attrition & static charge; can also granulate & coat |
| Vacuum dryer / Vacuum tray & rotary vacuum | Drying below atmospheric pressure → lower boiling point | Thermolabile, hygroscopic, oxidisable drugs; solvent recovery | Low temperature, no oxidation; expensive |
| Drum (film/ roller) dryer | Thin film of liquid dried on a heated rotating drum, scraped off | Slurries, pastes, milk, yeast, starch | Continuous, contact drying |
| Spray dryer | Liquid atomised into a hot air chamber → droplets dry instantly into spherical particles | ⭐ Thermolabile liquids: antibiotics, vaccines, enzymes, milk, blood products, spray-dried lactose | ⭐ Contact time is only a few seconds → drug stays cool by evaporative cooling; gives free-flowing spherical granules |
| Freeze dryer (Lyophiliser) | ⭐ Sublimation of ice under high vacuum, below the triple point | ⭐ Highly thermolabile & sterile products: vaccines, sera, antibiotics, plasma, enzymes, hormones, bacterial cultures, blood products | Stages: Freezing → Primary drying (sublimation) → Secondary drying (desorption). Product is porous, rapidly & completely soluble, moisture ~ 1 %. Most expensive & slowest, but preserves activity |
| Microwave/ dielectric dryer | Internal heating by microwaves (2450 MHz) | Granules, small batches | Rapid, uniform, no surface case-hardening |
| Infrared/ radiant dryer | Radiation from IR lamps | Thin layers, coatings | Surface drying |
| Turbo/ rotary dryer | Rotating shell with hot gas | Free-flowing granules, crystals | Continuous |
| Desiccator (chemical drying) | Moisture absorbed by a desiccant | Small quantities, crystals | Desiccants: silica gel, calcium chloride, phosphorus pentoxide, conc. H₂SO₄, calcium oxide, alumina |
| Sun/ air drying (natural) | Solar heat | Crude vegetable drugs | Cheap; risk of contamination & loss of volatile oil |

⭐ DRYING OF CRUDE DRUGS — special rules

- Digitalis, Belladonna, Ergot — dry rapidly below 60 °C to destroy enzymes and preserve glycosides.

- Volatile-oil-containing drugs (clove, peppermint, cinnamon) — dry in shade at low temperature to avoid loss of oil.

- Gentian & Vanilla — deliberately slow-dried/ fermented to develop constituents.

- Cinchona, Senna — dried in sun/ artificial heat.

- Aloe, Opium — dried by evaporation of juice/ latex.

- Final moisture usually < 5 % (up to 8–14 % for some).

🧠 MEMORY HOOK — "Which dryer for a thermolabile drug?"

Liquid & thermolabile → Spray dryer; extremely thermolabile & sterile → Freeze dryer; solid & thermolabile → Vacuum dryer; granules → Fluidised bed dryer.

## 3.6 EVAPORATION

📌 DEFINITION — Evaporation is the removal of a large amount of solvent (usually water) from a solution or suspension by conversion into vapour, so as to obtain a concentrated liquid product (the concentrate/ extract).

### Factors affecting the rate of evaporation

- Temperature (↑ temp → ↑ rate) and vapour pressure.

- Surface area exposed (↑ area → ↑ rate).

- Pressure (↓ pressure → ↓ boiling point → suitable for thermolabile drugs).

- Removal of vapour (air velocity/ vacuum, humidity).

- Viscosity, concentration & film thickness; scale/ crust formation; foaming.

- Heat transfer coefficient & agitation.

⭐ Boiling point elevation — as the solution concentrates, its boiling point rises; predicted by Dühring's rule (BP of solution vs BP of water is a straight line).

### 📋 Evaporators

| Equipment | Notes |
| --- | --- |
| Evaporating dish / pan (steam-jacketed pan) | Simple, open, small-scale; large surface; used for extracts, syrups |
| Evaporating still | Closed; permits solvent recovery (tinctures, alcohol) |
| Vacuum evaporator (vacuum still) | ⭐ Lower temperature → thermolabile drugs; faster; recovers solvent |
| Short-tube (calandria) / Vertical tube evaporator | Natural circulation; standard industrial |
| Long-tube (climbing/ falling film) evaporator | Rapid, single pass, short contact time — heat-sensitive liquids |
| Forced-circulation evaporator | Pump-driven, for viscous & scaling liquids |
| Multiple-effect evaporator | ⭐ Vapour of one effect is the heating medium of the next → great steam economy |
| Thermo-compression (vapour recompression) | Vapour compressed & reused |
| Rotary/ Wiped (agitated) thin-film evaporator | Very viscous & heat-sensitive materials |
| Rotary vacuum evaporator (lab) | Small-scale concentration of extracts |

⭐ In multiple-effect evaporators, the pressure decreases and the boiling point falls from the first to the last effect.

## 3.7 DISTILLATION

📌 DEFINITION — Distillation is the process of converting a liquid into its vapour by heating, and then condensing the vapour back to a liquid, in order to separate or purify it.

⚠️ Distillation vs Evaporation: in distillation the VAPOUR is collected (condensed) and is the product; in evaporation the vapour is discarded and the RESIDUE is the product.

### ⭐ Types of distillation

| Type | Principle | Used for | Examples |
| --- | --- | --- | --- |
| Simple distillation | Single vaporisation–condensation | Liquids with widely different boiling points; volatile from non-volatile | Distilled water (Purified Water IP) |
| Fractional distillation | Repeated vaporisation–condensation in a fractionating column | Liquids with close boiling points | Separation of alcohol–water, petroleum fractions, coal tar |
| Steam distillation | Steam passed through the material; immiscible volatile substance distils below 100 °C with the steam | ⭐ Volatile (essential) oils from crude drugs; heat-sensitive, water-immiscible substances | Clove oil, peppermint oil, eucalyptus oil, turpentine, aromatic waters |
| Vacuum (reduced-pressure) distillation | Boiling point lowered by reducing pressure | Thermolabile liquids | Vitamins, glycerin, fixed-oil fractions |
| Molecular distillation | Very high vacuum (10⁻³ mm Hg); molecules travel to a nearby cold surface without collision | ⭐ Vitamins A & E, hormones, high-molecular-weight thermolabile oils | Distance between evaporator & condenser < mean free path |
| Destructive distillation | Heating in the absence of air; substance decomposes | Wood tar, coal tar, birch tar, animal charcoal, methyl alcohol from wood | Product is a decomposition mixture |
| Extractive & azeotropic distillation | A third component alters relative volatility | Breaking azeotropes | Alcohol–water azeotrope (95.6 % ethanol, bp 78.15 °C) broken with benzene/ cyclohexane |
| Compound/ cohobation | Distillate returned & re-distilled from fresh drug | Concentrated aromatic waters (classical) | — |
| Flash distillation | Liquid flashes into vapour at reduced pressure | Petroleum | — |

🔢 NUMBERS — Purified water is stored not more than 24 h after collection for most uses; Water for Injection must be freshly distilled, pyrogen-free, collected in sterile, neutral-glass containers; first portion of distillate is discarded.

⭐ Distillation apparatus (parts) — still/ boiler (retort), still-head, condenser (Liebig, coil/ worm, reflux), receiver, thermometer, splash-head/ baffle to prevent entrainment. ⭐ Aromatic waters are made by steam distillation or by the solution/ trituration method (with talc as a distributing agent).

📌 RAOULT'S LAW — the partial vapour pressure of a component ∝ its mole fraction in the liquid. ⭐ Ideal mixtures obey it; deviation gives azeotropes (constant-boiling mixtures which cannot be separated by simple fractional distillation).

## 3.8 SUBLIMATION

📌 DEFINITION — Sublimation is the process in which a solid, on heating, is converted directly into vapour without passing through the liquid state, and the vapour on cooling condenses back into a solid (sublimate).

⭐ Sublimable substances (memorise): Camphor, Menthol, Thymol, Iodine, Naphthalene, Benzoic acid, Salicylic acid, Ammonium chloride, Sulphur (partly), Caffeine, Arsenious oxide, Hexamine, Anthracene, Solid CO₂ (dry ice).

- Uses: purification (camphor, iodine), identification test for balsams & benzoic acid, and freeze-drying is sublimation of ice under vacuum.

- Apparatus: sublimation is done in a shallow dish with an inverted funnel/ watch glass, or industrially in vacuum sublimation chambers.

- Triple point — the temperature & pressure at which solid, liquid & vapour coexist; sublimation occurs below the triple point.

## 3.9 EXTRACTION ⭐⭐ (very high-yield)

📌 DEFINITION — Extraction is the process of removing the soluble (desired) constituents of a crude drug with a suitable solvent, leaving behind the insoluble inert (marc).

📌 Menstruum = the solvent used for extraction. 📌 Marc = the inert, exhausted, insoluble residue of the drug left after extraction. 📌 Miscella = the extract solution (solvent + dissolved constituents). 📌 Percolate = the liquid that trickles out of a percolator. 📌 Colature = the strained liquid obtained after straining. 📌 Despumation = removal of scum/ frothy impurities from a liquid. 📌 Garbling = removal of foreign & unwanted parts from a crude drug after collection. 📌 Exhausted drug = drug from which active constituents have already been removed.

### ⭐ Choice of menstruum

| Menstruum | Extracts | Note |
| --- | --- | --- |
| Water | Gums, mucilages, sugars, salts, glycosides, tannins, some alkaloidal salts | Cheap; but supports microbial growth → needs preservative & has short shelf-life |
| Alcohol (ethanol) | Alkaloids (bases), resins, volatile oils, glycosides, tannins, chlorophyll | Self-preserving above ~ 20 % v/v (≥ 12–20 %); the standard menstruum of tinctures |
| Alcohol–water (hydro-alcoholic) | Widest range — most tinctures & fluid extracts | Strength chosen for each drug (e.g. 45 %, 60 %, 70 %, 90 %) |
| Glycerin | Tannins, prevents oxidation/ precipitation, sweet & preservative | Used in glycerin-containing extracts |
| Ether/ Chloroform/ Acetone/ Petroleum spirit | Fixed & volatile oils, resins, alkaloidal bases | Volatile, inflammable; used for oleoresins |
| Acidified water/ alcohol | Converts alkaloids to water-soluble salts | Dilute HCl/ acetic acid |
| Alkalinised solvent | Frees alkaloidal bases | Ammonia, lime |
| Diluted acetic acid | Vinegars (acetum) | Squill vinegar |

### 📋 METHODS OF EXTRACTION ⭐⭐

| Method | 📌 Definition & procedure | Used for | Example |
| --- | --- | --- | --- |
| Maceration | Drug is soaked in the whole of the menstruum in a closed vessel for a stated period (usually 7 days for organised, 2–7 days for unorganised), with occasional shaking, then strained, the marc pressed & the volume adjusted | Unorganised drugs, drugs whose active constituents are readily soluble; drugs that swell too much for percolation | Tincture of Orange peel, Tincture of Squill, Tincture of Lemon, Tincture of Tolu, Tincture of Benzoin, Compound Tincture of Benzoin, Tincture of Myrrh, Tincture of Ginger (weak) |
| — Simple maceration | Whole menstruum at once, 7 days, press marc | Organised drugs | Tincture of Squill |
| — Maceration with adjustment (for unorganised drugs) | Drug + menstruum, no pressing of marc (no marc), filtered & volume adjusted | Unorganised (gums, resins, balsams) | Tincture of Benzoin, Tincture of Tolu |
| — Multiple/ Circulatory maceration | Menstruum divided into portions used successively → more complete extraction | Concentrated preparations | Concentrated infusions, liquid extracts |
| — Digestion | ⭐ Maceration with GENTLE HEAT (about 35–40 °C, up to 50 °C), so long as the active constituents are not damaged | Drugs with tough/ hard tissue; increases solvent efficiency | Compound tincture of Benzoin (some texts), Mother tinctures |
| Percolation | ⭐ Drug packed in a percolator (a narrow, cone-shaped vessel) and the menstruum allowed to flow slowly downward through it, continuously exposing fresh drug to fresh solvent | ⭐ Organised drugs & potent drugs; the official method for most tinctures & liquid extracts | Tincture of Belladonna, Tincture of Nux vomica, Tincture of Cinchona, Tincture of Ipecac, Tincture of Digitalis, Liquid extract of Liquorice |
| — Steps of percolation | ⭐ (1) Size reduction → (2) Imbibition/ wetting (moistening with menstruum, keep 4 h in a closed vessel) → (3) Packing of the percolator → (4) Maceration in the percolator (24 h, outlet closed) → (5) Percolation proper (rate controlled) → (6) Final adjustment of volume | — | 🧠 "S-I-P-M-P-A" |
| — Rate of percolation | 🔢 Slow: 1 mL/min · Moderate: 1–3 mL/min · Rapid: 3–5 mL/min (per 1000 g of drug) | — | Slow percolation gives a more complete extraction |
| — Reserved percolation | ⭐ First ¾ of the percolate is RESERVED; the rest is percolated further, concentrated by evaporation, then added to the reserved portion and adjusted to volume — the reserved portion is never heated, so heat-labile & volatile constituents are protected | ⭐ Liquid extracts, potent & aromatic drugs | Liquid Extract of Belladonna, Liquid extract of Ipecac |
| — Continuous hot percolation (Soxhlet) | Solvent is boiled, condensed, percolates through drug, then siphons back to the flask — solvent is reused | ⭐ Small quantity of solvent, exhaustive extraction; not for thermolabile drugs | Oleoresins, fixed oil determination, alkaloid extraction |
| — Counter-current extraction | Drug & solvent move in opposite directions through a series of vessels | Industrial, very efficient | — |
| Infusion | 📌 A dilute aqueous solution of the readily soluble constituents of a crude drug, prepared by macerating the drug with COLD or BOILING water for a SHORT time | Drugs with readily soluble/ heat-labile constituents | Fresh Infusion of Senna, Infusion of Gentian (cold), Infusion of Digitalis |
| — Fresh infusion | 🔢 10 g drug + 100 mL water (1 in 10 or 5 %), macerated 15–30 min, strained. ⭐ Shelf-life only 12 hours | — | Prepared as required |
| — Concentrated infusion | 🔢 Prepared 8 times the strength of the fresh infusion, by maceration with alcohol (20–25 %) as a preservative; 1 part diluted with 7 parts water gives a fresh infusion | — | Concentrated infusion of Gentian, of Orange, of Quassia |
| Decoction | 📌 The drug is BOILED with water (usually 10 min) and strained while hot | Hard, woody drugs, barks, roots whose constituents are heat-stable & water-soluble | Decoction of Cinchona (classical); Ayurvedic kwath/ kadha |
| Hot continuous extraction | See Soxhlet above | — | — |
| Aqueous alcoholic fermentation | Drug fermented with water & jaggery | Ayurvedic Asava & Arishta | Draksharishta |
| Modern methods | Supercritical fluid extraction (SFE with CO₂), ultrasound-assisted (sonication), microwave-assisted (MAE), enzyme-assisted, phytonic (HFC solvent) extraction | Thermolabile & high-purity extracts | Volatile oils, caffeine decaffeination |
| Enfleurage / Expression | ⭐ Non-solvent methods for volatile oils: enfleurage = absorption of the perfume of flowers in fat; expression (cold pressing) = mechanical squeezing | Delicate flower oils; citrus peel oils (lemon, orange) | Lemon oil by expression |
| Ecuelle à piquer | Pricking & pressing of citrus peel | Citrus oils | — |

🔁 COMPARE — MACERATION vs PERCOLATION ⭐

| Feature | MACERATION | PERCOLATION |
| --- | --- | --- |
| Contact | Drug soaked in whole menstruum, static | Menstruum flows continuously through drug |
| Equilibrium | Extraction stops at equilibrium → incomplete | Fresh solvent always meets drug → complete/ exhaustive |
| Time | Long (2–7 days) | Shorter, controlled |
| Apparatus | Closed vessel/ jar | Percolator (conical, tapering) |
| Suited to | Unorganised drugs, drugs that swell, cheap drugs | Organised & potent drugs |
| Marc | Pressed (except with adjustment) | Not pressed; washed |
| Efficiency | Lower | Higher |
| Examples | Tinct. Benzoin, Tinct. Squill, Tinct. Orange | Tinct. Belladonna, Tinct. Nux vomica, Liquid extracts |

🔁 COMPARE — INFUSION vs DECOCTION

| Feature | INFUSION | DECOCTION |
| --- | --- | --- |
| Water temperature | Cold or boiling water poured on drug | Drug boiled in water |
| Duration | 15–30 min (short) | 10–15 min of boiling |
| Heat applied during process | No continued boiling | Continuous boiling |
| Drugs | Soft, heat-labile (leaves, flowers) | Hard, woody (roots, barks) |
| Strength | 1 in 10 (5–10 %) | Usually 1 in 20 concentrated to a set volume |
| Shelf life | ⭐ 12 hours | Very short (use fresh) |

⭐ CLASSICAL GALENICAL STRENGTHS — memorise! 🔢

| Galenical | Strength |
| --- | --- |
| Tincture (simple/ non-potent drug) | 1 in 5 (20 % w/v) — e.g. Tincture of Ginger, Orange, Lemon, Benzoin (1 in 5) |
| Tincture (potent drug) | 1 in 10 (10 % w/v) — e.g. Tinct. Belladonna, Nux vomica, Digitalis, Opium, Ipecac, Stramonium |
| Liquid (fluid) extract | ⭐ 1 in 1 — 1 mL ≡ 1 g of the drug |
| Fresh infusion | 1 in 10 |
| Concentrated infusion | 8 times the fresh infusion (1 : 7 dilution) |
| Syrup (simple syrup IP) | 66.7 % w/w sucrose (≈ 85 % w/v; 667 g sucrose in 1000 g syrup) |
| Spirit | Alcoholic/ hydro-alcoholic solution of a volatile substance, usually 5–10 % |
| Aromatic water | Saturated aqueous solution of a volatile oil (≈ 0.2 %) |
| Elixir | Sweetened, aromatic, hydro-alcoholic clear liquid |
| Glycerin (Glycerite) | Solution in glycerin (e.g. Glycerin of Boric acid, Glycerin of Starch) |
| Vinegar (Acetum) | Solution in dilute acetic acid |
| Linctus | Viscous, sweet, demulcent oral liquid for cough, taken undiluted, sipped slowly |
| Mucilage of Acacia | 35 % w/v |
| Liniment | Liquid/ semi-liquid for application to the skin with friction (rubbing) |
| Lotion | Liquid for application to the skin WITHOUT friction (dabbing) |
| Paint (Pigment) | Liquid applied to skin/ mucosa with a brush, leaves a film |
| Gargle | Concentrated aqueous solution to be diluted and used for the throat |
| Mouthwash | Aqueous solution for rinsing the mouth |
| Douche | Aqueous solution directed against a body cavity (eye, nose, vagina) |
| Enema | Liquid rectal administration (evacuant/ retention) |
| Collodion | Pyroxylin (nitrocellulose) in ether–alcohol; leaves a flexible film |
| Draught | A single-dose (usually 50 mL) mixture |

⚠️ TRAP — "1 in 1" is the LIQUID EXTRACT. Tinctures are 1 in 5 (simple) or 1 in 10 (potent). 🧠 "Potent = 10, Plain = 5, Liquid extract = 1."

## 3.10 FILTRATION & CLARIFICATION

📌 DEFINITION — Filtration is the process of separating suspended solid particles from a liquid or gas by passing it through a porous medium (filter) that retains the solid.

📌 Filtrate = the clear liquid that passes through. 📌 Cake/ residue = the retained solids. 📌 Clarification = removal of a small amount of solid to make a liquid sparkling clear. 📌 Straining = coarse separation through a cloth/ muslin.

### ⭐ Mechanisms of filtration

- Straining/ sieving — particles larger than the pore are held on the surface.

- Impingement — particles strike the medium and adhere.

- Entanglement — particles trapped in the mesh of fibres (cotton wool, filter pad).

- Attractive (electrostatic/ adsorption) forces — fine particles adsorbed on the medium (asbestos, charcoal, sintered glass, membrane).

- Autofiltration — the deposited cake itself acts as the filter medium.

### 🔢 Factors affecting filtration rate — POISEUILLE'S / DARCY'S equation

A × ΔP × t
V  =  ───────────────
        η × L × R

Rate ∝ area of filter (A), pressure difference (ΔP); Rate ∝ 1/viscosity (η), 1/thickness of cake (L), 1/resistance (R).

⭐ Hence, to increase the filtration rate: increase area & pressure (or apply vacuum), reduce viscosity by heating, keep the cake thin, use a filter aid, and pre-treat/ flocculate the slurry.

### Filter media & filter aids

| Filter media | Filter aids |
| --- | --- |
| Filter paper (Whatman), cotton wool, muslin/ flannel cloth, glass wool, asbestos pads, sintered glass & metal, porcelain (unglazed), ceramic candles, membrane filters (cellulose acetate/ nitrate), woven wire, synthetic fabrics | ⭐ Kieselguhr (diatomaceous earth), Talc, Purified siliceous earth, Charcoal, Paper pulp, Bentonite, Fuller's earth, Asbestos, Perlite |

⭐ Filter aid = an inert, insoluble, porous powder added to a slurry (or pre-coated on the filter) to prevent the pores being clogged by gelatinous/ slimy particles, thereby increasing the filtration rate. ⚠️ Never use a filter aid when the substance to be filtered off is the wanted product, or when it may adsorb the active drug (e.g. alkaloids on charcoal/ talc).

### 📋 Filtration equipment

| Equipment | Principle & use |
| --- | --- |
| Filter funnel + filter paper | Small-scale/ dispensing |
| Flannel bag / conical strainer | Straining of mixtures & syrups |
| Buchner funnel / Sintered glass with vacuum flask | Vacuum filtration, small scale |
| Filter press (plate-and-frame) | Batch, high pressure; viscous liquids, syrups, extracts; large area in small space; labour-intensive |
| Rotary drum (continuous vacuum) filter | Continuous, large volumes of slurry |
| Leaf filter (Sweetland/ Kelly/ Vallez) | Enclosed pressure filter; solvent-recovery friendly |
| Meta filter (edge filter) | Rings on a rod; needs a filter aid (kieselguhr); used for viscous liquids & clarifying syrups; ⭐ no filter medium needed, easy to clean, withstands high pressure |
| Cartridge filter | Disposable cartridge; clarification |
| Membrane filter | ⭐ 0.22 µm = sterilising (bacteria-proof) grade; 0.45 µm = for bioburden testing; used for sterility testing & sterilisation of thermolabile solutions |
| Seitz filter (asbestos pad) | Classical sterile filtration |
| Candle filters — Chamberland–Pasteur (porcelain), Berkefeld (kieselguhr), Mandler | Classical bacterial filtration |
| Sparkler filter | Small-scale clarification of syrups |
| HEPA filter | ⭐ Air filtration — removes ≥ 99.97 % of particles of 0.3 µm; used in LAF (laminar air flow) units & clean rooms |

🔢 REMEMBER: 0.22 µm (0.2 µm) membrane = sterilising filter; HEPA = 0.3 µm at 99.97 %; Clean-room Grade A/ Class 100 for aseptic filling.

## 3.11 CENTRIFUGATION

📌 DEFINITION — Centrifugation is the separation of two immiscible phases (or a solid from a liquid) by the application of a centrifugal force many times greater than gravity.

🔢 Centrifugal force F = m ω² r = m v²/r; Relative Centrifugal Force (RCF/ "g" value) = 1.118 × 10⁻⁵ × r (cm) × N² (rpm).

| Type | Use |
| --- | --- |
| Perforated-basket (filter/ centrifugal) centrifuge | Removing mother liquor from crystals, drying of wet solids |
| Non-perforated basket / Tubular bowl | Clarification of liquids, separating emulsions |
| Disc-bowl (Alfa-Laval) centrifuge | Continuous separation of two immiscible liquids (cream from milk, oil–water) |
| Supercentrifuge & Ultracentrifuge | Very fine particles, colloids, proteins, viruses, molecular weight determination |
| Laboratory (swinging-bucket/ angle-head) centrifuge | Blood, sediment volume of suspensions |
| Cyclone separator (gas) | Removing dust from an air stream |

⭐ Pharmaceutical uses: separation of crystals from mother liquor, clarification of syrups & injections, breaking or testing emulsions (accelerated stability), sedimentation-volume studies of suspensions, removing water from wet granules, plasma from blood.

## 3.12 CRYSTALLISATION

📌 DEFINITION — Crystallisation is the formation of solid crystals of a definite geometric form from a supersaturated solution or melt — used for purification and to obtain a product of defined size, shape & purity.

### Steps / requirements

- Supersaturation — achieved by (a) cooling, (b) evaporating solvent, (c) adding a miscible non-solvent (salting out), (d) chemical reaction, (e) altering pH.

- Nucleation — formation of nuclei (primary — homogeneous/ heterogeneous; secondary — induced by existing crystals/ seeding/ agitation).

- Crystal growth — deposition of solute on nuclei.

⭐ Rules of crystal size:

- Rapid cooling + vigorous agitation + high supersaturation → many nuclei → SMALL crystals.

- Slow cooling + no agitation + low supersaturation → few nuclei → LARGE crystals.

- Seeding with crystals of the desired form controls polymorph & size.

### Key terms

- Mother liquor — the liquid remaining after crystals are separated.

- Water of crystallisation — water chemically combined in the crystal (e.g. CuSO₄·5H₂O, Na₂CO₃·10H₂O, MgSO₄·7H₂O, FeSO₄·7H₂O, Alum·12H₂O).

- Efflorescence — loss of water of crystallisation to air.

- Polymorphism — same substance, different crystal forms, with different melting points, solubility & bioavailability (e.g. chloramphenicol palmitate — only polymorph B is active/absorbable; cocoa butter α, β, β′, γ forms; sulphathiazole).

- Pseudopolymorphism — solvates/ hydrates (e.g. ampicillin anhydrous vs trihydrate).

- Habit — external shape (tabular, prismatic, acicular/needle, plate).

- Amorphous form — no lattice; more soluble & faster dissolving but less stable than crystalline form. ⭐ Novobiocin: amorphous form is absorbed, crystalline is not.

Crystallisers: tank (batch) crystalliser, Swenson–Walker (continuous, scraped-surface), Krystal (Oslo) vacuum crystalliser, agitated batch vacuum crystalliser.

## 3.13 STERILISATION ⭐⭐

📌 DEFINITION — Sterilisation is the process by which all forms of living micro-organisms, including bacterial spores and viruses, are completely destroyed or removed from an article.

⚠️ Key distinctions (very frequently asked)

| Term | Meaning |
| --- | --- |
| Sterilisation | Complete destruction/ removal of ALL micro-organisms including spores |
| Disinfection | Destruction of vegetative pathogens (not necessarily spores), on inanimate objects |
| Antisepsis | Prevention of growth of micro-organisms on living tissue |
| Sanitisation | Reducing microbial numbers to a safe public-health level |
| Asepsis | Techniques that prevent the entry of micro-organisms |
| Bacteriostatic | Inhibits growth (reversible) |
| Bactericidal | Kills bacteria (irreversible) |
| Sporicidal | Kills spores |
| Pasteurisation | Partial sterilisation of liquids: 63 °C/ 30 min (holder/ LTLT) or 72 °C/ 15–20 s (flash/ HTST) |
| Tyndallisation | Intermittent (fractional) sterilisation — 100 °C for 20–60 min on 3 successive days |
| Depyrogenation | Destruction of pyrogens — needs dry heat 250 °C/ 30 min (or 180 °C/ 3–4 h) |

### 📋 THE CLASSIFICATION OF STERILISATION METHODS

STERILISATION
                              │
   ┌──────────┬───────────────┼────────────┬────────────┐
 PHYSICAL   CHEMICAL      MECHANICAL   RADIATION    ASEPTIC
   │           │          (Filtration)      │      TECHNIQUE
 ┌─┴───┐   Gaseous &        0.22 µm     Ionising &
DRY   MOIST  liquid                     Non-ionising
HEAT   HEAT  agents

### A. DRY HEAT STERILISATION 🔢

| Method | Conditions | Uses |
| --- | --- | --- |
| Hot air oven ⭐ | 160 °C for 2 h (also 170 °C/ 1 h; 180 °C/ 30 min) | Glassware, metal instruments, forceps, scissors, syringes, powders (talc, sulphur, ZnO), oils, fats, waxes, paraffin, glycerin, dry chemicals |
| Depyrogenation tunnel | 250 °C for 30 min | Destroying pyrogens on glass ampoules/ vials |
| Flaming | Direct flame | Inoculating loops, mouths of tubes |
| Incineration | Burning to ash | Contaminated dressings, animal carcasses, disposables |
| Red heat | Until red-hot | Wire loop, forceps tips, spatula |
| Infrared conveyor | 180 °C tunnel | Syringes, glass |

⭐ Mechanism of dry heat = OXIDATION / protein denaturation & charring, and it is LESS efficient than moist heat (needs higher temp & longer time) because dry air is a poor conductor and there is no latent heat.

### B. MOIST HEAT STERILISATION 🔢

| Method | Conditions | Uses |
| --- | --- | --- |
| Autoclave (saturated steam under pressure) ⭐⭐ | 121 °C at 15 psi (1.05 kg/cm²) for 15 min (also 115 °C/ 10 psi/ 30 min; 134 °C/ 30 psi/ 3 min) | ⭐ Aqueous injections, IV fluids, dressings, surgical instruments, rubber closures, culture media, glassware, gowns, filters |
| Boiling | 100 °C for 20–30 min (not sterilisation — spores survive) | Instruments (emergency disinfection) |
| Tyndallisation | 100 °C, 20–60 min × 3 successive days | Media & solutions containing heat-labile components (with spore germination in between) |
| Steam at 100 °C (Koch's/ Arnold's steamer) | Free-flowing steam 90 min | Media, egg/serum media |
| Low-temperature steam + formaldehyde (LTSF) | 73 °C + formaldehyde | Heat-sensitive rubber, plastics |
| Pasteurisation | 63 °C/30 min or 72 °C/15 s | Milk, vaccines |

⭐ Mechanism of moist heat = COAGULATION & DENATURATION of proteins by hydrolysis — efficient because of the latent heat of condensation of steam (steam gives up ~ 540 cal/g on condensing) and the high heat penetration.

⚠️ TRAP — Why is AIR removed from an autoclave? Because air is a poor conductor; entrapped air lowers the temperature at a given pressure and prevents steam penetration → sterilisation failure. Hence the air is displaced downward ("downward displacement autoclave") or removed by a vacuum pump (pre-vacuum autoclave).

⭐ AUTOCLAVE CONTROLS/ INDICATORS

| Type | Example |
| --- | --- |
| Physical | Thermocouple, temperature–pressure chart recorder, Browne's tube (green spot at 121 °C) |
| Chemical | Autoclave (Bowie–Dick) tape, chemical indicator strips |
| Biological ⭐ | Bacillus stearothermophilus (Geobacillus stearothermophilus) spores for moist heat/ autoclave; Bacillus subtilis var. niger (globigii) for dry heat & ethylene oxide; Bacillus pumilus for ionising radiation; Clostridium sporogenes for anaerobic |

🧠 MEMORY HOOK — biological indicators: "Stearothermophilus = Steam; Subtilis = Searing dry heat & ETO; Pumilus = Radiation."

🔢 F₀ VALUE — the equivalent time in minutes at 121 °C delivered to a product; pharmacopoeias require F₀ ≥ 8 (usually 12) for terminally sterilised aqueous products. 🔢 D-value — time required to reduce the microbial population by 90 % (1 log cycle) at a given temperature. 🔢 Z-value — the temperature change needed to alter the D-value by 1 log cycle (≈ 10 °C for moist heat). 🔢 Sterility Assurance Level (SAL) — probability of a non-sterile unit = 10⁻⁶.

### C. RADIATION STERILISATION

| Type | Source/ wavelength | Mechanism | Uses |
| --- | --- | --- | --- |
| Ionising — Gamma rays ⭐ | Cobalt-60 (also Caesium-137); dose 2.5 Mrad = 25 kGy | Ionisation → free radicals → DNA damage | ⭐ Disposable plastic syringes, catheters, sutures, gloves, surgical dressings, prepacked heat-labile items, some powders & ointments |
| Ionising — Electron beam (cathode rays) | Electron accelerator | Same | Surface sterilisation, thin packs; low penetration but fast |
| Non-ionising — UV light | 240–280 nm; most effective ≈ 253.7 nm (254 nm) from a mercury vapour lamp | Pyrimidine dimer formation in DNA | ⭐ Surface & air sterilisation of aseptic rooms, LAF benches, water; POOR penetration → NOT a true steriliser |
| Infrared | Heat radiation | Dry heat | Conveyor sterilisation of syringes |
| Microwave | 2450 MHz | Dielectric heating | Small-scale, moist materials |

⚠️ TRAP — UV light is only a surface disinfectant (cannot penetrate glass, water depth or paper); gamma radiation penetrates the final sealed pack and is therefore the method of choice for disposables.

### D. GASEOUS (CHEMICAL) STERILISATION

| Agent | Conditions | Uses | Notes |
| --- | --- | --- | --- |
| Ethylene oxide (ETO) ⭐ | ~ 500–1000 mg/L, 45–63 °C, 30–60 % RH, 4–6 h; usually diluted with CO₂/ N₂ (e.g. Carboxide 10 % ETO + 90 % CO₂) | Heat-labile plastics, catheters, sutures, powders, prosthetics, delicate instruments, syringes | Alkylating agent (alkylates DNA/protein). ⚠️ Flammable, explosive, mutagenic & carcinogenic; needs moisture to work; requires long degassing/ aeration to remove residues |
| Formaldehyde (gas/ fumigation) | 500 mL formalin/ 1000 ft³, 12–24 h; or LTSF | Rooms, aseptic areas, filters, wool, blankets | Irritant; neutralised with ammonia |
| Beta-propiolactone (BPL) | 0.2 % | Fumigation of rooms, vaccines | Carcinogenic |
| Hydrogen peroxide vapour / Plasma | Low temperature | Isolators, endoscopes, clean rooms | No toxic residue |
| Peracetic acid | 0.2–0.35 % | Isolators, endoscopes | — |
| Ozone | — | Water, surfaces | — |

### E. FILTRATION (MECHANICAL) STERILISATION

- ⭐ Membrane filter of 0.22 µm removes bacteria; 0.1 µm for mycoplasma; viruses are NOT removed (except by ultrafiltration).

- Used for thermolabile solutions: insulin, vaccines, sera, enzymes, hormones, vitamins, antibiotic solutions, eye drops, LVPs and for air (HEPA).

- Classical filters: Seitz (asbestos pad), sintered glass, Chamberland–Pasteur porcelain candle, Berkefeld (kieselguhr) candle, Mandler.

- Bubble-point test verifies membrane integrity.

### F. ASEPTIC TECHNIQUE

📌 A method by which a sterile product is prepared and filled in a sterile environment from already-sterilised components, when the product cannot itself be terminally sterilised.

- Requires clean rooms (Grade A/B, Class 100 laminar air flow), HEPA-filtered air, sterile garments, gloves, validated disinfection, media-fill validation.

- Used for antibiotic powders for injection, insulin, biologicals, ophthalmic ointments.

### G. CHOICE OF METHOD — the decision table ⭐

| Product | Method of choice |
| --- | --- |
| Aqueous injections, IV fluids, dressings, rubber closures | Autoclave, 121 °C/ 15 min |
| Glassware, metal instruments, powders, oils, fats, waxes, paraffin, glycerin | Dry heat (hot air oven), 160 °C/ 2 h |
| Thermolabile solutions (insulin, vaccines, enzymes, eye drops) | Membrane filtration (0.22 µm) + aseptic filling |
| Disposable plastics, syringes, catheters, sutures, gloves | Gamma radiation (2.5 Mrad) |
| Plastics & delicate instruments that cannot take radiation | Ethylene oxide |
| Clean-room air | HEPA filter + UV |
| Ampoule glass — pyrogen removal | Dry heat 250 °C/ 30 min |
| Culture media containing heat-labile sugars | Tyndallisation |

⭐ STERILITY TEST (IP) — carried out by membrane filtration or direct inoculation into Fluid Thioglycollate Medium (for anaerobes & aerobes, 30–35 °C) and Soyabean Casein Digest Medium (for fungi & aerobes, 20–25 °C), incubated 14 days. ⭐ PYROGEN TEST — rabbit method (rise in body temperature) and the in vitro LAL (Limulus Amoebocyte Lysate) test for bacterial endotoxins. ⭐ Pyrogens are lipopolysaccharides from the cell wall of Gram-negative bacteria; they are water-soluble, non-volatile, heat-stable and are NOT removed by autoclaving — they are removed by dry heat 250 °C, activated charcoal, ultrafiltration, or anion-exchange resins.

## 3.14 HEAT TRANSFER & ALLIED OPERATIONS

📌 Heat transfer is the flow of heat from a hotter to a cooler body. Three modes:

| Mode | Mechanism | Pharmaceutical example |
| --- | --- | --- |
| Conduction | Through a solid, molecule to molecule, no bulk movement | Heating an evaporating pan; hot plate |
| Convection | By actual movement of the heated fluid (natural or forced) | Hot-air oven, tray dryer, steam-jacketed pan |
| Radiation | Electromagnetic waves, no medium required | Infrared dryer, sun drying |

🔢 Fourier's law (conduction): Q = k A (ΔT)/L. 🔢 Newton's law of cooling / convection: Q = h A ΔT. 🔢 Stefan–Boltzmann (radiation): Q = σ A T⁴.

- Heat exchangers: shell-and-tube, double-pipe, plate, coil, jacketed vessel.

- Steam is the commonest heating medium (latent heat ≈ 540 cal/g); others: hot water, hot oil, Dowtherm, electricity.

- Humidification — adding moisture to air; dehumidification — removing it. 🔢 Dry-bulb temperature = actual air temperature; wet-bulb temperature = temperature of evaporative equilibrium; dew point = temperature at which air becomes saturated; relative humidity (RH) = (actual vapour pressure/ saturated vapour pressure) × 100. Read from the psychrometric (humidity) chart.

- ⭐ RH control is critical for effervescent granules, hygroscopic powders, capsules (35–45 % RH), and tablet compression.

## 3.15 📖 GLOSSARY OF CLASSICAL PHARMACEUTICAL OPERATIONS ⭐⭐

(The old "pharmaceutical processes and methods" list — pure one-mark material. Learn every definition.)

| Operation | 📌 Definition |
| --- | --- |
| Solution | Dissolving a solute in a solvent to form a homogeneous, molecularly dispersed system |
| Trituration | Rubbing/ grinding a substance in a mortar with a pestle (to reduce size or to mix) |
| Levigation | Grinding a solid with a small amount of liquid in which it is insoluble, to a smooth paste |
| Pulverisation by intervention | Size reduction with the aid of a volatile solvent that is subsequently removed (camphor + alcohol) |
| Spatulation | Mixing powders on a tile with a spatula, without pressure |
| Sifting/ Bolting | Passing powders through a sieve or bolting cloth |
| Tumbling | Mixing by rotating powders in a closed container |
| Geometric dilution | Mixing a potent drug with diluent by successive doubling |
| Elutriation | Separating fine from coarse particles by a stream of water or air |
| Decantation | Pouring off the clear supernatant liquid, leaving the sediment behind |
| Colature | The strained liquid; straining through cloth/muslin |
| Expression | Obtaining liquid by mechanical pressure/ squeezing (fixed oils, citrus peel oils, juice) |
| Clarification | Making a liquid sparkling clear by removing a small amount of suspended matter |
| Despumation | Removal of scum/ frothy impurities from the surface of a boiling liquid |
| Defecation | Removal of impurities from a liquid by allowing them to settle |
| Filtration | Separating solids from a liquid by a porous medium |
| Evaporation | Removing a large amount of solvent as vapour to give a concentrated liquid |
| Desiccation / Drying | Removing small amounts of moisture to give a dry solid |
| Exsiccation | ⭐ Removal of the water of crystallisation by heat, e.g. exsiccated ferrous sulphate, exsiccated alum, dried sodium sulphate |
| Efflorescence | Spontaneous loss of water of crystallisation to the atmosphere |
| Deliquescence | Absorption of atmospheric moisture until the solid liquefies |
| Calcination | ⭐ Heating a substance strongly, in the presence of air, to drive off volatile matter/ water & to bring about decomposition — e.g. calcined magnesia (MgO from MgCO₃), calcined calamine, lime from limestone |
| Ignition | Heating to redness, generally to constant weight (as in ash determination) |
| Torrefaction | ⭐ Roasting a drug until it is brown & brittle — e.g. roasted coffee, torrefied rhubarb, roasted asafoetida |
| Carbonisation | Converting organic matter to charcoal by heating without air |
| Destructive distillation | Heating in the absence of air so that the substance decomposes, and collecting the products (wood tar, coal tar) |
| Sublimation | Solid → vapour → solid on heating, without melting |
| Fusion | ⭐ Melting together two or more substances (with or without heat) — the basis of suppository & ointment preparation, and of eutectic mixtures |
| Deflagration | Rapid combustion with sparks/ flame, e.g. heating nitrates with charcoal |
| Lixiviation | ⭐ Leaching — extracting a soluble salt from a mixture by washing with water and evaporating (e.g. obtaining potash from wood ash) |
| Elixation/ Digestion | Extraction with the aid of gentle heat |
| Maceration | Soaking a drug in the whole of the menstruum in a closed vessel |
| Percolation | Slow downward passage of a menstruum through a packed column of the drug |
| Infusion | Short maceration of a drug with cold/ boiling water |
| Decoction | Boiling a drug with water and straining while hot |
| Precipitation | Formation of an insoluble solid from solution by a chemical reagent or a non-solvent |
| Salting out | Precipitating a substance by adding a large amount of a soluble salt/electrolyte |
| Crystallisation | Formation of geometrically definite solids from a supersaturated solution |
| Granulation | Converting fine powders into free-flowing granules (wet or dry) |
| Slugging | ⭐ Dry granulation by compressing powder into large tablets ("slugs") and re-milling them |
| Roller compaction | Dry granulation by passing powder between rollers |
| Emulsification | Dispersing one immiscible liquid in another with the aid of an emulgent |
| Homogenisation | Reducing globule size in an emulsion to make it uniform & stable |
| Nascent soap method | Forming the emulsifier (a soap) in situ — e.g. olive oil + lime water → calcium oleate (Carron oil/ Lime liniment) |
| Levigation (in ointments) | See above — removes grittiness |
| Adsorption | Retention of a substance on the surface of a solid (charcoal, kaolin) |
| Absorption | Uptake of one substance into the bulk of another |
| Dialysis | Separating crystalloids from colloids through a semi-permeable membrane |
| Osmosis / Reverse osmosis | Solvent flow through a semi-permeable membrane; RO used for water purification |
| Ion exchange (demineralisation) | ⭐ Removing ions by cation & anion exchange resins → de-ionised (demineralised) water |
| Deionisation vs Distillation | Deionised water is free of ions but may contain pyrogens & organics; distilled water is free of both |
| Saponification | Hydrolysis of a fat/oil by alkali to give soap + glycerol |
| Esterification | Acid + alcohol → ester |
| Hydrolysis | Splitting of a molecule by water |
| Oxidation & Reduction | Loss/ gain of electrons; important in drug degradation |
| Fermentation | Chemical change brought about by micro-organisms/ enzymes (alcohol, antibiotics, asava-arishta) |
| Maturation/ Ageing | Storage to allow desirable changes (tinctures, spirits) |
| Garbling | Removing foreign & unwanted parts from a collected crude drug |
| Blanching | Brief exposure to hot water/steam to inactivate enzymes |
| Dehydration | Complete removal of water |
| Insufflation | Blowing a powder into a body cavity |
| Nebulisation/ Atomisation | Breaking a liquid into a fine spray of droplets |
| Comminution | General term for size reduction |
| Trituration (2nd meaning) | ⭐ A 1 in 10 dilution of a potent drug with lactose (a "trituration" in homoeopathy & dispensing) |
| Attenuation/ Potentisation | Serial dilution & succussion (homoeopathy) |
| Percolate/ Marc/ Menstruum/ Miscella | See §3.9 |
| Sterilisation | Complete destruction of all micro-organisms including spores |
| Depyrogenation | Destruction/ removal of pyrogens |
| Lyophilisation | Freeze-drying — sublimation of ice under vacuum |
| Micronisation | Reducing particles to 1–10 µm (fluid energy mill) |
| Coating | Applying a layer (sugar, film, enteric) to a tablet |
| Encapsulation | Enclosing a drug in a shell (hard/ soft gelatin) |
| Microencapsulation | Coating minute particles/ droplets with a polymer film |
| Compression | Forming a tablet under pressure |
| Ampoule sealing | Tip-sealing or pull-sealing by fusion of glass |

## 3.16 CHAPTER 3 — RAPID REVISION SHEET

- Unit operation = physical change; unit process = chemical change.

- Rittinger → fine grinding (new surface); Kick → coarse (log of ratio); Bond → in between (work index).

- Fluid energy mill = micronising, thermolabile, sterile, no moving parts. Ball mill = impact + attrition, works at 50–80 % of critical speed, can grind wet/ dry & closed/ sterile. Hammer mill = impact, most versatile. Cutter mill = fibrous crude drugs. Colloid & triple roller mill = ointments/ emulsions. Edge & end runner = compression + shear.

- Dry grinding needs < 5 % moisture; wet grinding > 50 %.

- Sieve number = apertures per linear inch; higher number = finer powder.

- IP powder grades: Coarse 10/44, Mod. coarse 22/60, Mod. fine 44/85, Fine 85, Very fine 120; Microfine 90 % < 50 µm; Superfine 90 % < 10 µm.

- Positive/ negative/ neutral mixtures; convective, shear, diffusive mixing; geometric dilution for potent drugs; mix in ascending order of bulk density.

- Drying → product is a solid; Evaporation → product is a liquid.

- EMC cannot be removed; CMC ends the constant-rate period.

- Spray dryer = thermolabile liquids, contact time a few seconds. Freeze dryer (lyophiliser) = sublimation of ice under vacuum, for vaccines/ sera/ antibiotics. FBD = tablet granules, fast & uniform. Vacuum dryer = thermolabile solids.

- Steam distillation → volatile oils; molecular distillation → vitamins A & E; destructive distillation → tar; azeotrope of alcohol–water = 95.6 %.

- Menstruum = solvent; Marc = exhausted residue.

- Maceration 7 days (static, unorganised drugs); Percolation (continuous, organised & potent drugs). Percolation steps: size reduction → imbibition → packing → maceration 24 h → percolation → adjustment. Rates: slow 1, moderate 1–3, rapid 3–5 mL/min.

- Reserved percolation protects heat-labile constituents (reserve ¾ of percolate).

- Soxhlet = continuous hot percolation, small solvent volume.

- Tincture: simple 1 in 5, potent 1 in 10. Liquid extract 1 in 1. Fresh infusion 1 in 10, keeps 12 h. Concentrated infusion = 8× strength (dilute 1 + 7). Simple syrup = 66.7 % w/w sucrose. Mucilage of acacia 35 % w/v.

- Liniment = rubbed on; Lotion = dabbed on; Paint = brushed on; Gargle = diluted for throat.

- Filtration rate ∝ area × pressure ÷ (viscosity × cake thickness × resistance).

- Filter aid (kieselguhr/ talc) prevents clogging — never use if the solid is the product.

- 0.22 µm membrane = sterilising filter; HEPA = 99.97 % of 0.3 µm; Meta filter needs kieselguhr.

- Rapid cooling + agitation → small crystals; slow cooling → large crystals.

- Polymorphism affects solubility & bioavailability (chloramphenicol palmitate form B).

- Autoclave: 121 °C, 15 psi, 15 min (moist heat = coagulation of protein). Hot air oven: 160 °C, 2 h (dry heat = oxidation). Depyrogenation: 250 °C, 30 min. Gamma radiation: 2.5 Mrad (25 kGy), Cobalt-60, for disposables. UV: 254 nm, surface only. ETO: alkylating agent, needs moisture, for heat-labile plastics.

- Biological indicators: B. stearothermophilus → autoclave; B. subtilis → dry heat & ETO; B. pumilus → radiation.

- F₀ ≥ 8–12; D-value = 1 log reduction; Z-value ≈ 10 °C; SAL = 10⁻⁶.

- Pyrogens = lipopolysaccharides of Gram-negative bacteria; heat-stable; removed by dry heat 250 °C, charcoal, ultrafiltration; detected by rabbit test & LAL test.

- Sterility test media: Fluid Thioglycollate (30–35 °C) & Soyabean Casein Digest (20–25 °C), 14 days.

- Exsiccation = removal of water of crystallisation. Calcination = strong heating in air. Torrefaction = roasting. Lixiviation = leaching a soluble salt with water. Fusion = melting together. Despumation = removing scum. Colature = strained liquid. Expression = squeezing out liquid. Decantation = pouring off supernatant.

Next → Chapter 4: General Directions on Dispensing, Weighing & Measuring